SECTION 087100

DOOR HARDWARE

- GENERAL

      - GENERAL PROVISIONS

        - Attention is directed to the CONTRACT AND GENERAL CONDITIONS and all Sections within DIVISION 01 - GENERAL REQUIREMENTS which are hereby made a part of this Section of the Specifications.

      - DESCRIPTION OF WORK

        - Work Included:  Provide labor, materials and equipment necessary to complete the work of this Section, including but not limited to the following:

          - Cylindrical locksets.

          - Exit devices.

          - Mortise locksets.

          - Door closers.

          - Hinges.

          - Architectural trim.

          - Deadbolts.

          - Sliding and pocket door hardware.

          - Barn door hardware.

          - Tubular interior lever sets.

          - Auxiliary hardware.

        - Related Work: The following items are not included in this Section and are specified under the designated Sections:

          - Section 081110 - HOLLOW METAL Doors and Frames for astragals provided as part of a fire-rated labeled assembly and for door silencers provided as part of the frame.

          - Section 081400 - FLUSH Wood Doors for astragals provided as part of a fire-rated labeled assembly.

          - Section 083310 - Overhead Coiling Doors for overhead coiling door hardware, except cylinders.

          - Section 083320 - Overhead Coiling Grilles for overhead coiling grille hardware, except cylinders.

          - Section 083610 - Sectional Doors for sectional door hardware, except cylinders.

          - Section 084113 - Aluminum-Framed Entrances and Storefronts for entrance door hardware, except cylinders.

          - Section 102210 - Wire Mesh Partitions for wire mesh partition door hardware, except cylinders.

          - Division 26 - Electrical for power for electrified door hardware and interface with building control and security systems.

      - REFERENCE STANDARDS

        - Comply with the following reference standards and publications as applicable to the project..

        - American National Standards Institute (ANSI):

          - ANSI A117.1  Accessible and Usable Buildings and Facilities.

        - Builders Hardware Manufacturers Association (BHMA):

          - ANSI/BHMA A156.1 Butts and Hinges.

          - ANSI/BHMA A156.2 Bored and Preassembled Locks and Latches.

          - ANSI/BHMA A156.3 Exit Devices.

          - ANSI/BHMA A156.4 Door Controls - Closers.

          - ANSI/BHMA A156.5 Auxiliary Locks and Associated Products.

          - ANSI/BHMA A156.6 Architectural Door Trim.

          - ANSI/BHMA A156.7 Template Hinge Dimensions.

          - ANSI/BHMA A156.8 Door Controls - Overhead Stops and Holders.

          - ANSI/BHMA A156.13 Mortise Locks and Latches.

          - ANSI/BHMA A156.14 Sliding and Folding Door Hardware.

          - ANSI/BHMA A156.16 Auxiliary Hardware.

          - ANSI/BHMA A156.17 Self-Closing Hinges and Pivots.

          - ANSI/BHMA A156.18 Materials & Finishes.

          - ANSI/BHMA A156.25 Electrified Locking Devices.

          - ANSI/BHMA A156.28 Recommended Practices for Mechanical Keying Systems.

          - ANSI/BHMA A156.29 Exit Locks, Exit Locks with Exit Alarms, Exit Alarms, Alarms for Exit.

          - ANSI/BHMA A156.30 High Security Cylinders.

          - ANSI/BHMA A156.115 Hardware Preparation in Steel Doors with Steel Frames.

          - ANSI/BHMA A156.115W Hardware Preparation in Wood Doors with Wood or Steel Frames.

        - Door and Hardware Institute (DHI):

          - ANSI/DHI A115.IG  Installation Guide for Doors and Hardware.

          - DHI Keying Systems and Nomenclature.

          - DHI Sequence and Format for the Hardware Schedule.

        - International Building Code (IBC).

        - National Fire Protection Association (NFPA):

          - NFPA 80  Fire Doors and Fire Windows.

          - NFPA 252  Fire Tests of Door Assemblies.

        - Underwriters Laboratories Inc. (UL):

          - UL 10C  Positive Pressure Fire Tests Of Door Assemblies.

          - UL 305  Panic Hardware.

          - UL 437  Drill and Pick Resistant Key Cylinders.

          - UL 1034  Burglary-Resistant Electric Locking Mechanisms.

      - SUBMITTALS

        - Product Data:  Include installation details, material descriptions, dimensions of individual components and profiles, and finishes.

        - Shop Drawings:  Details of electrified door hardware, indicating the following:

          - Wiring Diagrams:  Detail wiring for power, signal, and control systems and differentiate between manufacturer-installed and field-installed wiring.  Include the following:

            - System schematic.

            - Point-to-point wiring diagram.

            - Riser diagram.

            - Elevation of each door.

          - Detail interface between electrified door hardware and fire alarm access control and building control and security systems.

        - Door Hardware Schedule:  Prepared by or under the supervision of supplier, detailing fabrication and assembly of door hardware, as well as procedures and diagrams.  Coordinate the final Door Hardware Schedule with doors, frames, and related work to ensure proper size, thickness, hand, function, and finish of door hardware.

          - Format:  Comply with scheduling sequence and vertical format in DHI's "Sequence and Format for the Hardware Schedule."

          - Organization:  Organize the Door Hardware Schedule into door hardware sets indicating complete designations of every item required for each door or opening.

            - Organize door hardware sets in same order as in the Door Hardware Schedule at the end of Part 3.

          - Content:  Include the following information:

            - Type, style, function, size, label, hand, and finish of each door hardware item.

            - Manufacturer of each item.

            - Fastenings and other pertinent information.

            - Location of each door hardware set, cross-referenced to Drawings, both on floor plans and in door and frame schedule.

            - Explanation of abbreviations, symbols, and codes contained in schedule.

            - Mounting locations for door hardware.

            - Door and frame sizes and materials.

            - Description of each electrified door hardware function, including location, sequence of operation, and interface with other building control systems.

              - Sequence of Operation:  Include description of component functions that occur in the following situations:  authorized person wants to enter; authorized person wants to exit; unauthorized person wants to enter; unauthorized person wants to exit.

          - Submittal Sequence:  Submit the final Door Hardware Schedule at earliest possible date, particularly where approval of the Door Hardware Schedule must precede fabrication of other work that is critical in the Project construction schedule.  Include Product Data, Samples, Shop Drawings of other work affected by door hardware, and other information essential to the coordinated review of the Door Hardware Schedule.

        - Keying Schedule:  Prepared by or under the supervision of supplier, detailing the Owner's final keying instructions for locks.  Include schematic keying diagram and index each key set to unique door designations.

        - Product Certificates:  Signed by manufacturers of electrified door hardware certifying that products furnished comply with requirements.

          - Certify that door hardware approved for use on types and sizes of labeled fire doors complies with listed fire door assemblies.

        - Qualification Data:  For firms and persons specified in "Quality Assurance" Article.

          - Include lists of completed projects with project names and addresses of architects and owners, and other information specified.

        - Maintenance Data:  For each type of door hardware to include in maintenance manuals specified in Division 01.

        - Warranties:  Special warranties specified in this Section.

      - QUALITY ASSURANCE

        - Installer Qualifications:  An experienced installer who has completed door hardware similar in material, design, and extent to that indicated for this Project and whose work has resulted in construction with a record of successful in-service performance.

        - Supplier Qualifications:  Door hardware supplier with warehousing facilities in Project's vicinity and who is or employs a qualified Architectural Hardware Consultant, available during the course of the Work to consult with Contractor, Designer, and the Owner about door hardware and keying.

          - Electrified Door Hardware Supplier Qualifications:  An experienced door hardware supplier who has completed projects with electrified door hardware similar in material, design, and extent to that indicated for this Project, whose work has resulted in construction with a record of successful in-service performance, and who is acceptable to manufacturer of primary materials.

            - Engineering Responsibility:  Prepare data for electrified door hardware, including Shop Drawings, based on testing and engineering analysis of manufacturer's standard units in assemblies similar to those indicated for this Project.

          - Scheduling Responsibility:  Preparation of door hardware and keying schedules.

        - Architectural Hardware Consultant Qualifications:  A person who is currently certified by the Door and Hardware Institute as an Architectural Hardware Consultant and who is experienced in providing consulting services for door hardware installations that are comparable in material, design, and extent to that indicated for this Project.

          - Electrified Door Hardware Qualifications:  Experienced in providing consulting services for electrified door hardware installations.

        - Source Limitations:  Obtain each type and variety of door hardware from a single manufacturer, unless otherwise indicated.

          - Provide electrified door hardware from same manufacturer as mechanical door hardware, unless otherwise indicated.  Manufacturers that are listed to perform electrical modifications, by a testing and inspecting agency acceptable to authorities having jurisdiction, are acceptable.

        - Regulatory Requirements:  Comply with provisions of the following:

          - Where indicated to comply with accessibility requirements, comply with Massachusetts Architectural Access Board and the Americans with Disabilities Act (ADA), "Accessibility Guidelines for Buildings and Facilities (ADAAG),"  as follows:

            - Handles, Pulls, Latches, Locks, and other Operating Devices:  Shape that is easy to grasp with one hand and does not require tight grasping, tight pinching, or twisting of the wrist.

            - Door Closers:  Comply with the following maximum opening-force requirements indicated:

              - Interior Hinged Doors:  5 lbf applied perpendicular to door.

              - Sliding or Folding Doors:  5 lbf applied parallel to door at latch.

              - Fire Doors:  Minimum opening force allowable by authorities having jurisdiction.

            - Thresholds:  Not more than 1/2 inch high.  Bevel raised thresholds with a slope of not more than 1:2.

          - NFPA 101:  Comply with the following for means of egress doors:

            - Latches, Locks, and Exit Devices:  Not more than 15 lbf to release the latch.  Locks shall not require the use of a key, tool, or special knowledge for operation.

            - Delayed-Egress Locks:  Lock releases within 15 seconds after applying a force not more than 15 lbf for not more than 3 seconds.

            - Door Closers:  Not more than 30 lbf to set door in motion and not more than 15 lbf to open door to minimum required width.

            - Thresholds:  Not more than 1/2 inch high.

          - Electrified Door Hardware:  Listed and labeled as defined in NFPA 70, Article 100, by a testing agency acceptable to authorities having jurisdiction.

        - Fire-Rated Door Assemblies:  Provide door hardware for assemblies complying with NFPA 80 that are listed and labeled by a testing and inspecting agency acceptable to authorities having jurisdiction, for fire ratings indicated, based on testing according to NFPA 252.

          - Test Pressure:  Test at atmospheric pressure.

        - Keying Conference:  Conduct conference at Project site to comply with requirements in Division 01.  Incorporate keying conference decisions into final keying schedule after reviewing door hardware keying system including, but not limited to, the following:

          - Function of building, flow of traffic, purpose of each area, degree of security required, and plans for future expansion.

          - Preliminary key system schematic diagram.

          - Requirements for key control system.

          - Address for delivery of keys.

        - Preinstallation Conference:  Conduct conference at Project site to comply with requirements in Division 01. Review methods and procedures related to electrified door hardware including, but not limited to, the following:

          - Inspect and discuss electrical roughing-in and other preparatory work performed by other trades.

          - Review sequence of operation for each type of electrified door hardware.

          - Review and finalize construction schedule and verify availability of materials, Installer's personnel, equipment, and facilities needed to make progress and avoid delays.

          - Review required testing, inspecting, and certifying procedures.

      - DELIVERY, STORAGE, AND HANDLING

        - Delivery:  Inventory door hardware on receipt and provide secure lock-up for door hardware delivered to Project site.  Tag each item or package separately with identification related to the final Door Hardware Schedule, and include basic installation instructions with each item or package.

        - Keys:  Deliver keys to manufacturer of key control system.

      - COORDINATION

        - Coordination:  Coordinate layout and installation of recessed pivots and closers with floor construction.  Cast anchoring inserts into concrete.  Concrete, reinforcement, and formwork requirements are specified in Section 033000 - CAST-IN-PLACE CONCRETE.

        - Templates:  Obtain and distribute to the parties involved templates for doors, frames, and other work specified to be factory prepared for installing door hardware.  Check Shop Drawings of other work to confirm that adequate provisions are made for locating and installing door hardware to comply with indicated requirements.

        - Electrical System Roughing-in:  Coordinate layout and installation of electrified door hardware with connections to power supplies, fire alarm system and detection devices, access control and building control system.

      - WARRANTY

        - General Warranty:  Special warranties specified in this Article shall not deprive the Owner of other rights the Owner may have under other provisions of the Contract Documents and shall be in addition to, and run concurrent with, other warranties made by Contractor under requirements of the Contract Documents.

        - Special Warranty:  Written warranty, executed by manufacturer agreeing to repair or replace components of door hardware that fail in materials or workmanship within specified warranty period.  Failures include, but are not limited to, the following:

          - Structural failures including excessive deflection, cracking, or breakage.

          - Faulty operation of operators and door hardware.

          - Deterioration of metals, metal finishes, and other materials beyond normal weathering.

        - Warranty Period:  Three years from date of Substantial Completion, unless otherwise indicated.

        - Warranty Period for Manual Closers:  Ten years from date of Substantial Completion.

      - MAINTENANCE SERVICE

        - Maintenance Tools and Instructions:  Furnish a complete set of specialized tools and maintenance instructions as needed for the Owner's continued adjustment, maintenance, and removal and replacement of door hardware.

- PRODUCTS

      - MANUFACTURER

        - Basis-of-Design Manufacturer: Unison Hardware, Inc., INOX, Tel:  916-388-1888. Fax:  916-388-8322. Internet:  www.inoxproducts.com.

      - CYLINDRICAL LOCKSETS

        - Basis of Design:  INOX; CL Series, Grade 1, Cylindrical Lock:

          - Rating: [Non-fire-rated] [Fire-rated].

          - Trim: [07 Lever] [09 Lever] [06 Lever].

          - Cylinder:  [Standard] [Small format interchangeable cylinder less core].

          - Function:  [10 Passage Latch] [40 Bath/Bedroom Privacy Lock] [17 Dummy Trim] [50 Office Lock] [53 Entrance Lock] [70 Classroom Lock] [80 Storeroom Lock] [50IC Office Lock] [53IC Entrance Lock] [70IC Classroom Lock] [80IC Storeroom Lock].

          - Backset:  [2-3/4 inches] [2-3/8 inches].

          - Finish:  [26D Satin Chrome] [3 Bright Brass] [4 Satin Brass] [10B Oil Rubbed Bronze] [19G Black] [26 Bright Chrome].

        - Basis of Design:  INOX; BL Series, Grade 2, Cylindrical Lock:

          - Trim: [07 Lever] [09 Lever] [06 Lever].

          - Function:  [10 Passage Latch] [40 Bath/Bedroom Privacy Lock] [25 Exit/Communicating Lock] [17 Dummy Trim] [50 Office Lock] [53 Entrance] [70 Classroom Lock] [80 Storeroom Lock] [50IC Office Lock] [53IC Entrance Lock] [70IC Classroom Lock] [80IC Storeroom Lock].

          - Clutch:  [Non-clutch] [Clutch].

          - Finish:  [26D Satin Chrome] [3 Bright Brass] [4 Satin Brass] [10B Oil Rubbed Bronze] [19G Black] [26 Bright Chrome].

      - EXIT DEVICES

        - Basis of Design:  INOX; ED70 RIM Series:

          - Device: [Panic exit] [Fire exit].

          - Type: Rim.

          - Trim: [07 Lever] [09 Lever] [Cylindrical trim] [Escutcheon trim] [Thumb-piece and plate trim] [Wing pull].

          - Function:  [Entry] [Night Latch] [Passage] [Dummy].

          - Size: [36 inch] [48 inch].

          - Dogging:  [None] [Cylinder dogging].

          - Finish:  [26D Satin Chrome] [AL Aluminum Painted] [DU Duro/Dark Bronze Painted] [GD Gold Painted] [3 Bright Brass] [4 Satin Brass] [10B Oil Rubbed Bronze] [19G Black] [32D Satin Stainless Steel].

        - Basis of Design:  INOX; ED70 SVR Series:

          - Device: [Panic exit] [Fire exit].

          - Type: Surface mounted vertical rod.

          - Trim: [07 Lever] [09 Lever] [Cylindrical trim] [Escutcheon trim] [Thumb-piece and plate trim] [Wing pull].

          - Function:  [Entry] [Night Latch] [Passage] [Dummy].

          - Size: [36 x 84 inches] [36 x 96 inches] [48 x 84 inches] [48 x 96 inches].

          - Dogging:  [None] [Cylinder dogging].

          - Finish:  [26D Satin Chrome] [AL Aluminum Painted] [DU Duro/Dark Bronze Painted] [GD Gold Painted] [3 Bright Brass] [4 Satin Brass] [10B Oil Rubbed Bronze] [19G Black] [32D Satin Stainless Steel].

        - Basis of Design:  INOX; ED70 LBR Series:

          - Device: [Panic exit] [Fire exit].

          - Type: Less bottom rod.

          - Trim: [07 Lever] [09 Lever] [Cylindrical trim] [Escutcheon trim] [Thumb-piece and plate trim] [Wing pull].

          - Function:  [Entry] [Night Latch] [Passage] [Dummy].

          - Size: [36 x 84 inches] [36 x 96 inches] [48 x 84 inches] [48 x 96 inches].

          - Dogging:  [None] [Cylinder dogging].

          - Finish:  [26D Satin Chrome] [AL Aluminum Painted] [DU Duro/Dark Bronze Painted] [GD Gold Painted] [3 Bright Brass] [4 Satin Brass] [10B Oil Rubbed Bronze] [19G Black] [32D Satin Stainless Steel].

        - Basis of Design:  INOX; ED81 RIM Series:

          - Device: [Panic exit] [Fire exit].

          - Type: Rim.

          - Trim: [07 Lever] [09 Lever] [Cylindrical trim] [Escutcheon trim] [Thumb-piece and plate trim] [Wing pull].

          - Function:  [Classroom] [Night Latch] [Passage] [Dummy].

          - Size: [36 inches] [48 inches].

          - Dogging:  [None] [Cylinder dogging].

          - Finish:  [26D Satin Chrome] [AL Aluminum Painted] [DU Duro/Dark Bronze Painted] [GD Gold Painted] [3 Bright Brass] [4 Satin Brass] [10B Oil Rubbed Bronze] [19G Black] [32D Satin Stainless Steel].

        - Basis of Design:  INOX; ED81 SVR Series:

          - Device: [Panic exit] [Fire exit].

          - Type: Surface mounted vertical rod.

          - Trim: [07 Lever] [09 Lever] [Cylindrical trim] [Escutcheon trim] [Thumb-piece and plate trim] [Wing pull].

          - Function:  [Classroom] [Night Latch] [Passage] [Dummy].

          - Size: [36 x 84 inches] [36 x 96 inches] [48 x 84 inches] [48 x 96 inches].

          - Dogging:  [None] [Cylinder dogging].

          - Finish:  [26D Satin Chrome] [AL Aluminum Painted] [DU Duro/Dark Bronze Painted] [GD Gold Painted] [3 Bright Brass] [4 Satin Brass] [10B Oil Rubbed Bronze] [19G Black] [32D Satin Stainless Steel].

        - Basis of Design:  INOX; ED81 CVR Series:

          - Device: [Panic exit] [Fire exit].

          - Type: Concealed vertical rod.

          - Trim: [07 Lever] [09 Lever] [Cylindrical trim] [Escutcheon trim] [Thumb-piece and plate trim].

          - Function:  [Classroom] [Night Latch] [Passage] [Dummy].

          - Size: [36 x 84 inches] [36 x 96 inches] [48 x 84 inches] [48 x 96 inches].

          - Dogging:  [None] [Cylinder dogging].

          - Finish:  [26D Satin Chrome] [AL Aluminum Painted] [DU Duro/Dark Bronze Painted] [GD Gold Painted] [3 Bright Brass] [4 Satin Brass] [10B Oil Rubbed Bronze] [19G Black] [32D Satin Stainless Steel].

        - Basis of Design:  INOX; ED93 RIM Series:

          - Device: [Panic exit] [Fire exit].

          - Type: Rim.

          - Trim: [07 Lever] [09 Lever] [Escutcheon trim] [Cylindrical trim] [Thumb-piece and plate trim] [Designer Trim, X-Series, 201] [Designer Trim, X-Series, 202] [Designer Trim, X-Series, 205] [Designer Trim, X-Series, 221] [Designer Trim, X-Series, 242] [Designer Trim, X-Series, 244] [Designer Trim, X-Series, 318] [Designer Trim, X-Series, 345] [Designer Trim, X-Series, 346] [Designer escutcheon trim].

          - Function:  [Entry] [Night Latch] [Passage] [Dummy].

          - Size: [36 inch] [48 inch].

          - Included Options:  [None] [Cylinder dogging] [Motor driven quiet latch retraction] [Device with request to exit switch] [Device with latch bolt monitor switch].

          - Finish:  [26D Satin Chrome] [AL Aluminum Painted] [DU Duro/Dark Bronze Painted] [GD Gold Painted] [3 Bright Brass] [4 Satin Brass] [10B Oil Rubbed Bronze] [19G Black] [32D Satin Stainless Steel].

        - Basis of Design:  INOX; ED93 SVR Series:

          - Device: [Panic exit] [Fire exit].

          - Type: Surface mounted vertical rod.

          - Trim: [07 Lever] [09 Lever] [Escutcheon trim] [Thumb-piece and plate trim] [Designer Trim, X-Series, 201] [Designer Trim, X-Series, 202] [Designer Trim, X-Series, 205] [Designer Trim, X-Series, 221] [Designer Trim, X-Series, 242] [Designer Trim, X-Series, 244] [Designer Trim, X-Series, 318] [Designer Trim, X-Series, 345] [Designer Trim, X-Series, 346] [Designer escutcheon trim].

          - Function:  [Entry] [Night Latch] [Passage] [Dummy].

          - Size: [36 x 84 inches] [36 x 96 inches] [48 x 84 inches] [48 x 96 inches].

          - Included Options:  [None] [Cylinder dogging] [Motor driven quiet latch retraction] [Device with request to exit switch] [Device with latch bolt monitor switch].

          - Finish:  [26D Satin Chrome] [AL Aluminum Painted] [DU Duro/Dark Bronze Painted] [GD Gold Painted] [3 Bright Brass] [4 Satin Brass] [10B Oil Rubbed Bronze] [19G Black] [32D Satin Stainless Steel].

        - Basis of Design:  INOX; ED93 LBR Series:

          - Device: [Panic exit] [Fire exit].

          - Type: Less bottom rod.

          - Trim: [07 Lever] [09 Lever] [Escutcheon trim] [Thumb-piece and plate trim] [Designer Trim, X-Series, 201] [Designer Trim, X-Series, 202] [Designer Trim, X-Series, 205] [Designer Trim, X-Series, 221] [Designer Trim, X-Series, 242] [Designer Trim, X-Series, 244] [Designer Trim, X-Series, 318] [Designer Trim, X-Series, 345] [Designer Trim, X-Series, 346] [Designer escutcheon trim].

          - Function:  [Entry] [Night Latch] [Passage] [Dummy].

          - Size: [36 x 84 inches] [36 x 96 inches] [48 x 84 inches] [48 x 96 inches].

          - Included Options:  [None] [Cylinder dogging] [Motor driven quiet latch retraction] [Device with request to exit switch] [Device with latch bolt monitor switch].

          - Finish:  [26D Satin Chrome] [AL Aluminum Painted] [DU Duro/Dark Bronze Painted] [GD Gold Painted] [3 Bright Brass] [4 Satin Brass] [10B Oil Rubbed Bronze] [19G Black] [32D Satin Stainless Steel].

        - Basis of Design:  INOX; ED93 MED Series:

          - Device: [Panic exit] [Fire exit].

          - Type: [MED Mortise exit device, exit only] [MED08 Mortise exit device, entry] [MED09 Mortise exit device, storeroom] [MED14 Mortise exit device, passage].

          - Trim: [07 Lever] [09 Lever] [Escutcheon trim] [Designer Trim, X-Series, 201] [Designer Trim, X-Series, 202] [Designer Trim, X-Series, 205] [Designer Trim, X-Series, 221] [Designer Trim, X-Series, 242] [Designer Trim, X-Series, 244] [Designer Trim, X-Series, 318] [Designer Trim, X-Series, 345] [Designer Trim, X-Series, 346] [Designer escutcheon trim].

          - Function:  [Entry] [Night Latch] [Passage] [Dummy].

          - Size: [36 inch] [48 inch].

          - Included Options:  [None] [Cylinder dogging] [Motor driven quiet latch retraction] [Device with request to exit switch] [Device with latch bolt monitor switch].

          - Finish:  [26D Satin Chrome] [AL Aluminum Painted] [DU Duro/Dark Bronze Painted] [GD Gold Painted] [3 Bright Brass] [4 Satin Brass] [10B Oil Rubbed Bronze] [19G Black] [32D Satin Stainless Steel].

        - Basis of Design:  INOX; ED93F-[CVRM] [CVRW] Series:

          - Device: [Panic exit] [Fire exit].

          - Type: Concealed vertical rod for [metal doors] [wood doors].

          - Trim: [07 Lever] [09 Lever] [Escutcheon trim] [Designer Trim, X-Series, 202] [Designer Trim, X-Series, 205] [Designer Trim, X-Series, 221] [Designer Trim, X-Series, 242] [Designer Trim, X-Series, 244] [Designer Trim, X-Series, 318] [Designer Trim, X-Series, 345] [Designer Trim, X-Series, 346] [Designer escutcheon trim].

          - Function:  [Entry] [Night Latch] [Passage] [Dummy].

          - Size: [36 x 84 inches] [36 x 96 inches] [48 x 84 inches] [48 x 96 inches].

          - Included Options:  [None] [Cylinder dogging] [Motor driven quiet latch retraction] [Device with request to exit switch] [Device with latch bolt monitor switch].

          - Finish:  [26D Satin Chrome] [AL Aluminum Painted] [DU Duro/Dark Bronze Painted] [GD Gold Painted] [3 Bright Brass] [4 Satin Brass] [10B Oil Rubbed Bronze] [19G Black] [32D Satin Stainless Steel].

      - MORTISE LOCKSETS

        - Basis of Design:  INOX; Commercial Series Grade 1:

          - Trim:  [02 Lever] [07 Lever] [09 Lever] {RS Round Sectional] [NP Narrow Plate Escutcheon] [PE Wide Plate Escutcheon].

          - Function:  [F01 Passage] [F22 Privacy] [F22 Privacy with Coin-Turn] [F22 Privacy with Coin-Turn Indicator] [F19 Privacy with Deadbolt] [F19 Privacy with Dead Bolt and Coin-Turn] [Privacy with Dead Bolt and Coin-Turn Electrically Unlocked, Fail Secure] [F19 Privacy with Dead Bolt and Coin-Turn Indicator] [F31 Exit/Communicating] [Half Dummy Trim without Lock Case] [Full Dummy Trim without Lock Case] [Half Dummy Trim with Lock Case] [Full Dummy Trim with Lock Case] [F04 Office and Inner Entry] [F05 Classroom] [F07 Storeroom] [Storeroom Electrically Locked, Fail Safe] [Storeroom Electrically Unlocked, Fail Secure] [Storeroom Electrically Locked with Request to Exit Switch, Fail Safe] [Storeroom Electrically Unlocked with Request to Exit Switch, Fail Secure] [F20 Entrance with Toggle] [Entrance without Toggle] [Entrance without Toggle Electrically Locked, Fail Safe] [Entrance without Toggle Electrically Unlocked, Fail Secure] [F13 Dormitory/Exit/Corridor] [Closet/Storeroom] [F21 Dormitory with Deadbolt] [Storeroom Lock with Deadbolt] [F15 Entrance/Hotel/Restroom Lock] [Apartment Entrance/Public Restroom] [F32 Classroom Security Intruder Latchbolt] [F30 Asylum/Institution Lock] [Institution Electrically Locked, Fail Safe] [Institution Electrically Unlocked, Fail Secure] [F33 Classroom Security Lock] [F14 Store Door Lock] [Institution Lock with Deadbolt] [[F17 Deadlock, Cylinder and Thumb-turn] [F16 Deadlock, Double Cylinder] [F29 Deadlock, Classroom] [F18 Deadlock, Single Cylinder] [Deadlock, Thumb-turn Only] [Deadlock, Thumb-turn x Coin-Turn Release] [Deadlock, Thumb-turn x Coin-Turn Release and Indicator].

          - Finish:  [26D Satin Chrome] [3 Bright Brass] [4 Satin Brass] [15 Satin nickel] [10B Oil Rubbed Bronze] [32 Bright stainless steel] [32D Satin stainless steel] [19G Black] [26 Bright Chrome].

        - Basis of Design:  INOX; X Series Designer, Sectional:

          - Trim:  [101 Cologne] [102 Munich] [103 Oslo] [104 Brussels] [105 Frankfurt] [106 Copenhagen] [107 Stockholm] [108 Vienna] [109 Stuttgart] [210 Air Stream] [211 Breeze] [213 Cabernet] [214 Champagne] [217 Horizon] [221 Aurora] [223 Phoenix] [225 Waterfall] [226 Summer] [227 Stratus] [243 Sunrise] [244 Twilight] [251 Sequoia] [311 Crest] [318 Café] [344 Ecco] [345 Tokyo] [346 Osaka] [351 Toronto] [379 Arctic] [380 Polaris].

          - Lever:  [201 Cologne] [202 Munich] [205 Frankfurt] [221 Aurora] [243 Sunrise] [244 Twilight] [318 Café] [345 Tokyo] [346 Osaka].

          - Function:  F01 Passage] [F22 Privacy] [F22 Privacy with Coin-Turn] [F22 Privacy with Coin-Turn Indicator] [F19 Privacy with Deadbolt] [F19 Privacy with Dead Bolt and Coin-Turn] [Privacy with Dead Bolt and Coin-Turn Electrically Unlocked, Fail Secure] [F19 Privacy with Dead Bolt and Coin-Turn Indicator] [F31 Exit/Communicating] [Half Dummy Trim without Lock Case] [Full Dummy Trim without Lock Case] [Half Dummy Trim with Lock Case] [Full Dummy Trim with Lock Case] [F04 Office and Inner Entry] [F05 Classroom] [F07 Storeroom] [Storeroom Electrically Locked, Fail Safe] [Storeroom Electrically Unlocked, Fail Secure] [Storeroom Electrically Locked with Request to Exit Switch, Fail Safe] [Storeroom Electrically Unlocked with Request to Exit Switch, Fail Secure] [F20 Entrance with Toggle] [Entrance without Toggle] [Entrance without Toggle Electrically Locked, Fail Safe] [Entrance without Toggle Electrically Unlocked, Fail Secure] [F13 Dormitory/Exit/Corridor] [Closet/Storeroom] [F21 Dormitory with Deadbolt] [Storeroom Lock with Deadbolt] [F15 Entrance/Hotel/Restroom Lock] [Apartment Entrance/Public Restroom] [F32 Classroom Security Intruder Latchbolt] [F30 Asylum/Institution Lock] [Institution Electrically Locked, Fail Safe] [Institution Electrically Unlocked, Fail Secure] [F33 Classroom Security Lock] [F14 Store Door Lock] [Institution Lock with Deadbolt] [[F17 Deadlock, Cylinder and Thumb-turn] [F16 Deadlock, Double Cylinder] [F29 Deadlock, Classroom] [F18 Deadlock, Single Cylinder] [Deadlock, Thumb-turn Only] [Deadlock, Thumb-turn x Coin-Turn Release] [Deadlock, Thumb-turn x Coin-Turn Release and Indicator].

          - Finish:  [3 Bright Brass] [4 Satin Brass] [5 Antique Brass] [10 Satin Light Bronze] [10B Oil Rubbed Bronze] [14 Bright Nickel] [15 Satin Nickel] [15A Satin Nickel Blackened] [19 Flat Black] [26 Polished Chrome] [26D Satin Chrome] [32 Bright stainless steel] [32D Satin stainless steel] [SP Two-tone satin and bright stainless steel] [19G Graphite black] [PVD4 Satin brass] [C19G Ceramic Graphite black] [C19 Ceramic Flat black] [CGY Ceramic Stormy gray] [CDB Ceramic Dark bronze].

        - Basis of Design:  INOX; X Series Designer, SF and LA Escutcheon:

          - Trim:  [101 Cologne] [102 Munich] [103 Oslo] [104 Brussels] [105 Frankfurt] [106 Copenhagen] [107 Stockholm] [108 Vienna] [109 Stuttgart] [210 Air Stream] [211 Breeze] [213 Cabernet] [214 Champagne] [217 Horizon] [221 Aurora] [223 Phoenix] [225 Waterfall] [226 Summer] [227 Stratus] [243 Sunrise] [244 Twilight] [251 Sequoia] [311 Crest] [318 Café] [344 Ecco] [345 Tokyo] [346 Osaka] [351 Toronto] [379 Arctic] [380 Polaris].

          - Lever:  [201 Cologne] [202 Munich] [205 Frankfurt] [221 Aurora] [243 Sunrise] [244 Twilight] [318 Café] [345 Tokyo] [346 Osaka].

          - Function:  F01 Passage] [F22 Privacy] [F22 Privacy with Coin-Turn] [F22 Privacy with Coin-Turn Indicator] [F19 Privacy with Deadbolt] [F19 Privacy with Dead Bolt and Coin-Turn] [Privacy with Dead Bolt and Coin-Turn Electrically Unlocked, Fail Secure] [F19 Privacy with Dead Bolt and Coin-Turn Indicator] [F31 Exit/Communicating] [Half Dummy Trim without Lock Case] [Full Dummy Trim without Lock Case] [Half Dummy Trim with Lock Case] [Full Dummy Trim with Lock Case] [F04 Office and Inner Entry] [F05 Classroom] [F07 Storeroom] [Storeroom Electrically Locked, Fail Safe] [Storeroom Electrically Unlocked, Fail Secure] [Storeroom Electrically Locked with Request to Exit Switch, Fail Safe] [Storeroom Electrically Unlocked with Request to Exit Switch, Fail Secure] [F20 Entrance with Toggle] [Entrance without Toggle] [Entrance without Toggle Electrically Locked, Fail Safe] [Entrance without Toggle Electrically Unlocked, Fail Secure] [F13 Dormitory/Exit/Corridor] [Closet/Storeroom] [F21 Dormitory with Deadbolt] [Storeroom Lock with Deadbolt] [F15 Entrance/Hotel/Restroom Lock] [Apartment Entrance/Public Restroom] [F32 Classroom Security Intruder Latchbolt] [F30 Asylum/Institution Lock] [Institution Electrically Locked, Fail Safe] [Institution Electrically Unlocked, Fail Secure] [F33 Classroom Security Lock] [F14 Store Door Lock] [Institution Lock with Deadbolt] [[F17 Deadlock, Cylinder and Thumb-turn] [F16 Deadlock, Double Cylinder] [F29 Deadlock, Classroom] [F18 Deadlock, Single Cylinder] [Deadlock, Thumb-turn Only] [Deadlock, Thumb-turn x Coin-Turn Release] [Deadlock, Thumb-turn x Coin-Turn Release and Indicator].

          - Finish:  [3 Bright Brass] [4 Satin Brass] [5 Antique Brass] [10 Satin Light Bronze] [10B Oil Rubbed Bronze] [14 Bright Nickel] [15 Satin Nickel] [15A Satin Nickel Blackened] [19 Flat Black] [26 Polished Chrome] [26D Satin Chrome] [32 Bright stainless steel] [32D Satin stainless steel] [SP Two-tone satin and bright stainless steel] [19G Graphite black] [PVD4 Satin brass] [C19G Ceramic Graphite black] [C19 Ceramic Flat black] [CGY Ceramic Stormy gray] [CDB Ceramic Dark bronze].

      - DOOR CLOSERS

        - Basis of Design:  INOX; DC61 Series, Door Closer:

          - Power Size: [1-4] [2-6] adjustable.

          - Function:  Standard.

          - Cover:  [Standard] [Less cover].

          - Finish:  [AL Aluminum Painted] [DU Duro/Dark Bronze Painted] [GD Gold Painted].

        - Basis of Design:  INOX; DC85 Series, Grade 1 Heavy Duty Door Closer:

          - Power Size: 1-6 adjustable.

          - Function:  [Standard] [Delayed action] [Friction hold open] [Cush arm] [Hold open cush arm] [Extra duty arm] [Hold open extra duty arm] [Slide track arm] [Slide track hold open arm] [Spring cush].

          - Cover:  [Plastic, full cover] [Plastic, slim cover] [Stainless steel] [Less cover].

          - Finish:  [AL Aluminum Painted] [DU Duro/Dark Bronze Painted] [GD Gold Painted] [3 Bright Brass] [4 Satin Brass] [15 Satin nickel] [32D Satin stainless steel] [19G Black] [26 Bright Chrome].

        - Basis of Design:  INOX; DC90 Series, Grade 1 Extra Heavy Duty Door Closer:

          - Power Size: 1-6 adjustable.

          - Function:  [Standard] [Delayed action] [Friction hold open] [Cush arm] [Hold open cush arm] [Extra duty arm] [Hold open extra duty arm] [Slide track arm] [Slide track hold open arm] [Spring cush].

          - Cover:  [Plastic, full cover] [Stainless steel cover].

          - Finish:  [AL Aluminum Painted] [DU Duro/Dark Bronze Painted] [GD Gold Painted] [3 Bright Brass] [4 Satin Brass] [15 Satin nickel] [32D Satin stainless steel] [19G Black] [26 Bright Chrome].

      - HINGES

        - Basis of Design:  INOX; HG Series:

          - Type: [Full surface] [Half surface] [Full mortise] [Half mortise] [Standard, Plain Bearing] [Standard, Two Anti-Friction Ball Bearing] [Standard Weight NRP (Non-Rising Pin)] [Heavyweight, Four Anti-Friction Ball Bearing] [Heavyweight NRP (Non-Rising Pin)] [Standard Weight, Ball Bearing, Radius Corner] [Spring Hinge] [Spring Hinge, Radius Corner] [Power Transfer Hinges].

          - Material:  [Steel] [Stainless steel] [Brass].

          - Size:  [4x4 inches] [4-1/2 x 4-1/2 inches] [4-1/2 x 4 inches] [5 x 4-1/2 inches] [5 x 5 inches].

          - Finish:  [3 Bright Brass] [4 Satin Brass] [5 Antique Brass] [10 Satin Light Bronze] [10B Oil Rubbed Bronze] [14 Bright Nickel] [15 Satin Nickel] [15A Satin Nickel Blackened] [19 Flat Black] [26 Polished Chrome] [26D Satin Chrome] [32 Bright stainless steel] [32D Satin stainless steel].

      - ARCHITECTURAL TRIM

        - Kickplates:

          - Material:  [Type 304 Stainless steel] [Type 400 Stainless steel].

          - Size:  [6 x 34 inches] [8 x 34 inches] [10 x 34 inches] [10 x 46 inches] [12 x 34 inches] [36 x 34 inches].

          - Finish:  32D Satin stainless steel.

        - Mop Plate:

          - Material:  [Type 304 Stainless steel] [Type 400 Stainless steel].

          - Size:  6 x 34 inches.

          - Finish:  32D Satin stainless steel.

        - Armor Plate:

          - Material:  [Type 304 Stainless steel] [Type 400 Stainless steel].

          - Size:  36 x 34 inches.

          - Finish:  32D Satin stainless steel.

        - ADA Door Pulls:

          - Shape:  8 inch OA.

          - Material:  Stainless steel.

          - Finish:  [32D Satin stainless steel] [32 Bright stainless steel] [19G Black] [10B Oil Rubbed Bronze].

        - Door Pulls:

          - Shape:  [T-shape] [U-shape] [C-shape] [Rectangular].

          - Material:  Stainless steel.

          - Finish:  [32D Satin stainless steel] [32 Bright stainless steel] [19G Black] [10B Oil Rubbed Bronze].

        - Push and Pull Plates:

          - Type:  [Push plate] [Plate with round pulls].

          - Material:  Stainless steel.

          - Size:  4 x 16 inches.

          - Finish:  [32D Satin stainless steel] [3 Bright Brass] [4 Satin Brass] [10B Oil Rubbed Bronze] [19G Black] [26D Brushed chrome].

      - DEADBOLTS

        - Basis of Design:  INOX; BX Series, Grade 2 Standard Duty Commercial Deadbolt:

          - Door Thickness:  1-3/8 inches to 2 inches.

          - Backset:  [2-3/8 inches] [2-3/4 inches].

          - Exposed Trim:  Solid cast alloy.

          - Deadbolt:  1 inch throw.

          - Function:  [Single cylinder deadbolt] [Double cylinder deadbolt] [Door bolt only] [Classroom deadbolt] [Cylinder and blank plate].

          - Finish:  [26D Satin Chrome] [3 Bright Brass] [4 Satin Brass] [15 Satin nickel] [10B Oil Rubbed Bronze] [19G Black] [26 Bright Chrome].

        - Basis of Design:  INOX; CX Series, Grade 1 Heavy Duty Commercial Deadbolt:

          - Door Thickness:  1-9/16 inches to 2 inches.

          - Backset:  [2-3/8 inches] [2-3/4 inches].

          - Exposed Trim:  Solid brass.

          - Deadbolt:  1 inch throw.

          - Function:  [Single cylinder deadbolt] [Double cylinder deadbolt] [Door bolt only] [Classroom deadbolt] [Cylinder and blank plate].

          - Finish:  [26D Satin Chrome] [3 Bright Brass] [4 Satin Brass] [15 Satin nickel] [10B Oil Rubbed Bronze] [19G Black] [26 Bright Chrome].

      - SLIDING, POCKET, AND BARN DOOR HARDWARE

        - Basis of Design:  INOX; PD8000 Series:

          - Function:  [Privacy] [Single Cylinder Entry] [Patio with thumbturn only] [Passage inactive] [Passage active].

          - Backset:  [2 inches] [2-1/2 inches] [2-3/4 inches].

          - Trims for Doors minimum 1-3/4 inch thick:  [FH27 Linear] [FH29 Arc] FH31 Regal] [FH32 Crown] [FH22 Luna] [FH23 Urban].

          - Trim for Doors 1-3/8 inch thick:  [FH17 Bezel (Wrought)] [FH27SL Linear (Cast)].

          - Cylinder:  [15/16 inches] [1 inch] [1-1/8 inches] [1-1/4 inches] [1-3/8 inches].

          - Finish:  [3 Bright Brass] [4 Satin Brass] [14 Bright Nickel] [15 Satin Nickel] [19G Black] [26 Polished Chrome] [26D Satin Chrome] [32 Bright stainless steel] [32D Satin stainless steel] [38 Aged Brass] [SP Split 32 and 32D] [10B Oil Rubbed Bronze] [C19G Graphite black] [C19 Flat black] [CGY Stormy gray] [CDB Dark bronze].

        - Basis of Design:  INOX; PD5000 Series:

          - Function:  [Privacy] [Single Cylinder Entry] [Patio with thumbturn only] [Passage inactive] [Passage active].

          - Backset:  [2 inches] [2-1/2 inches] [2-3/4 inches].

          - Pull Style:  [C-Shape PHIX30108] [Ladder Pull PHIX330.512.8] [U-shape PHIX31108] [Rectangular PHIX45714].

          - Cylinder:  [15/16 inches] [1 inch] [1-1/8 inches] [1-1/4 inches] [1-3/8 inches].

          - Finish:  [4 Satin Brass] [32 Bright stainless steel] [32D Satin stainless steel] [10B Oil Rubbed Bronze] [C19G Graphite black] [C19 Flat black] [CGY Stormy gray] [CDB Dark bronze] [19G Black].

        - Basis of Design:  INOX; PD95 Series Emergency Egress Sliding Door Lock:

          - Function:  [Exit/Communication] [Privacy with ADA thumbturn and release] [Single Cylinder Entry] [Patio with ADA thumbturn only] [Single Cylinder Classroom].

          - Backset:  [2-1/2 inches] [2-3/4 inches].

          - Rose Style:  [XGT Round] [XGS Square].

          - Lever: [201 Cologne] [202 Munich] [205 Frankfurt] [221 Aurora] [243 Sunrise] [244 Twilight] [345 Tokyo] [346 Osaka].

          - Finish:  [10B Oil Rubbed Bronze] [19G Black] [32 Bright stainless steel] [32D Satin stainless steel] [C19G Graphite black] [C19 Flat black] [CGY Stormy gray] [CDB Dark bronze].

        - Basis of Design:  INOX; PD96 Series Self Latching Mortise Sliding Door Lock:

          - Function:  [Passage] [Office] [Communicating/Patio] [Storeroom] [Privacy] [Entry] [Classroom].

          - Backset:  [2-1/2 inches] [2-3/4 inches].

          - Rose Style:  [XGT Round] [XGS Square].

          - Lever:  [201 Cologne] [202 Munich] [205 Frankfurt] [221 Aurora] [243 Sunrise] [244 Twilight] [345 Tokyo] [346 Osaka].

          - Finish:  [10B Oil Rubbed Bronze] [19G Black] [32 Bright stainless steel] [32D Satin stainless steel] [C19G Graphite black] [C19 Flat black] [CGY Stormy gray] [CDB Dark bronze].

        - Basis of Design:  INOX; PD97 Series Electrified Mortise Sliding Door Lock:

          - Function:  [Privacy] [Entry/Office] [Classroom].

          - Options:  [PD97PT] [PD97ES] [REX] [DBM] [ATL] [DPS].

          - Backset:  [2-1/2 inches] [2-3/4 inches].

          - Rose Style:  [XGT Round] [XGS Square].

          - Lever:  [201 Cologne] [202 Munich] [205 Frankfurt] [221 Aurora] [243 Sunrise] [244 Twilight] [345 Tokyo] [346 Osaka].

          - Finish:  [10B Oil Rubbed Bronze] [19G Black] [32D Satin stainless steel] [C19G Graphite black] [C19 Flat black] [CGY Stormy gray] [CDB Dark bronze].

        - Basis of Design:  INOX; PD2000 Series TwistLock Edge-Pull Privacy Lock:

          - Function:  [Privacy Lock with Edge Pull] [Passage Edge Pull].

          - Backset:  2-3/8 inches.

          - Trim:  [Round FH12] [Square FH13] [Round Concealed Fastener FH22CF] [Square Concealed Fastener FH23CF].

          - Finish:  [32D Satin stainless steel] [19G Graphite black] [PVD4 Satin brass] [10B Oil Rubbed Bronze] [32 Bright stainless steel] [C19G Graphite black] [C19 Flat black] [CGY Stormy gray] [CDB Dark bronze].

        - Thumbturn and Release:  [GR02] [GR04] [GR07] [SR02] [SR04] [ADA TT07].

        - Flush Pulls, Cast:

          - Style:  [FH27] [FH31] [FH22] [FH32] [FH29] [FH23] [FH22CF] [FH23CF].

          - Finish:  [3 Bright Brass] [4 Satin Brass] [14 Bright Nickel] [15 Satin Nickel] [26 Polished Chrome] [26D Satin Chrome] [32 Bright stainless steel] [32D Satin stainless steel] [38 Aged Brass] [SP Split 32 and 32D] [10B Oil Rubbed Bronze] [C19G Graphite black] [C19 Flat black] [CGY Stormy gray] [CDB Dark bronze].

        - Flush Pulls, FH17 Series, Wrought:

          - Style:  [FH1700] [FH1703] [FH1704] [FH1792].

          - Finish:  [19G Graphite black] [32D Satin stainless steel].

        - Flush Cups:

          - Style:  [FHIX01] [FHIX02] [FHIX03] [FHIX04] [FHIX05] [FHIX06] [FHIX07].

          - Finish:  [19G Black] [32 Bright stainless steel] [32D Satin stainless steel] [10B Oil Rubbed Bronze] [4 Satin Brass] [C19G Graphite black] [C19 Flat black] [CGY Stormy gray] [CDB Dark bronze].

      - BARN DOOR HARDWARE

        - Basis of Design:  INOX; BD4000 Series Privacy Lock for Sliding Barn Doors:

          - Trim:  [EC1216 Round escutcheon with standard thumbturn] [EC1217 Round escutcheon with ADA thumbturn] [EC1314 Square escutcheon with thumbturn] [EC1315 Rectangle escutcheon with ADA thumbturn] [EC1515 Rectangular escutcheon with rectangular thumbturn] [EC1517 Rectangular escutcheon with ADA thumbturn] [FH22CF Luna concealed trim with thumbturn] [FH23CF Urban concealed trim with thumbturn].

          - Backset:  [2-1/4 inches] [3-1/2 inches].

          - Finish:  [32 Bright stainless steel] [32D Satin stainless steel] [10B Oil Rubbed Bronze] [PVD4] [4 Satin bronze] [19G Graphite black] [C19G Graphite black] [C19 Flat black] [CGY Stormy gray] [CDB Dark bronze].

        - Basis of Design:  INOX; BD1000 Series Privacy Lock for Sliding Barn Doors:

          - Trim:  [Standard BD1022] [ADA BD1023].

          - Finish:  [32 Bright stainless steel] [32D Satin stainless steel] [10B Oil Rubbed Bronze] [19G Graphite black] [PVD4 Satin brass] [C19G Graphite black] [C19 Flat black] [CGY Stormy gray] [CDB Dark bronze] [26D Satin Chrome].

      - TUBULAR INTERIOR LEVERSETS

        - Basis of Design:  INOX; Tubular Interiors-Designer Interior Leversets, Grade 2:

          - Trim:  [Standard Round Rose RA] [Standard Square Rose SE] [Round GT] [Square GS] [Oval Plate LA] [Rectangular Plate SF] [Rectangular Plate SH].

          - Function:  [Passage] [Privacy] [Single Dummy] [Half Passage].

          - Style:  [Cologne] Munich] [Oslo] [Brussels] [Frankfurt] [Copenhagen] [Stockholm] [Vienna] [Stuttgart] [Air-Stream] [Breeze] [Cabernet] [Champagne] [Horizon] [Aurora] [Phoenix] [Waterfall] [Summer] [Stratus] [Sunrise] [Twilight] [Sequoia] [Crest] [Café] [Ecco] [Tokyo] [Osaka] [Toronto] [Polaris] [Arctic].

          - Latch:  [TL4] [TL4F] [TL7].

          - Material:  Stainless steel.

          - Backset:  [2-3/8 inches] [2-3/4 inches].

          - Finish:  [10B Oil Rubbed Bronze] [19G Graphite black] [32 Polished stainless steel] [32D Satin stainless steel] [SP Split] [PVD4] [C19G Graphite black] [C19 Flat black] [CGY Stormy gray] [CDB Dark bronze].

      - AUXILIARY HARDWARE

        - Coordinator and Accessories

          - Coordinator:  [COR60] [COR72] [COR84].

          - Mounting Bracket:  [MB434] [MB414].

            - Jamb Depth:  [4-3/4 inches] [4-1/4 inches].

            - Stop Height:  5/8 inches.

            - Stop Width:  [7/8 to 2-1/4 inches] [Over 2-1/4 inches].

          - Carry Bar CB100:  Mounted on inactive door to engage coordinator hold open mechanism.

          - Finish:  [AL Aluminum Painted] [DU Duro/Dark Bronze Painted].

        - Door Guard:  DG128.

          - Maximum 4 inch door opening when engaged.

          - Finish:  [26D Satin Chrome] [3 Bright Brass] [4 Satin Brass] [10B Oil Rubbed Bronze] [19G Graphite black].

        - Door Stops and Bumpers:

          - Type:  [Low dome stop DS436] [High dome stop DS438] [Concave door stop, wall mount DS401] [Convex door stop, wall mount DS406].

          - Finish:  [26D Satin Chrome] [32D Satin stainless steel] [3 Bright Brass] [4 Satin Brass] [10B Oil Rubbed Bronze] [19G Graphite black].

        - Door Holder:  DH235.

          - Cast brass door holder.

          - Finish:  [26D Satin Chrome] [3 Bright Brass] [4 Satin Brass] [10B Oil Rubbed Bronze] [19G Graphite black] [32D Satin stainless steel].

        - Door Viewer: DV33200 F Fire Rated 200 degree.

          - Polished glass lens.

          - Finish:  [26D Satin Chrome] [3 Bright Brass] [4 Satin Brass] [10B Oil Rubbed Bronze] [15 Satin Nickel] [19G Graphite black].

        - Dust Proof Strike: DPS280.

          - Material:  Extruded brass.

          - Plunger:  Spring loaded, 3/4 inch.

          - Finish:  [26D Satin Chrome] [3 Bright Brass] [4 Satin Brass] [10B Oil Rubbed Bronze] [19G Graphite black].

        - Manual Flushbolts:

          - Type:  [FB196W Wood] [FB197M Metal].

          - Material:  Forged brass.

          - Throw:  5/8 inches.

          - Vertical Adjustment:  7/8 inches.

          - Lever:  Spring loaded.

          - Finish:  [26D Satin Chrome] [3 Bright Brass] [4 Satin Brass] [10B Oil Rubbed Bronze] [19G Graphite black].

        - Automatic Flushbolts:

          - Type:  [Metal doors FBA292M] [Wood doors FBA291W].

          - Material:  Steel body and stainless steel components.

          - Throw:  3/4 inches.

          - Vertical Adjustment:  [3/4 inches] [1 inch].

          - Lever:  Spring loaded.

          - Finish:  [19G Black] [26D Satin Chrome] [3 Bright Brass] [4 Satin Brass] [10B Oil Rubbed Bronze].

        - Magnetic Door Holder: DHM2100.

          - Extension:  [1 inch] [1/2 inch] [3/4 inch].

          - Finish:  [28 Aluminum painted] [32D Satin stainless steel] [19G Graphite black].

        - Roller Latches:

          - Type:  [RL300, 1 x 3-3/8 x 1-5/8 inches] [RL320, 1-1/8 x 2-1/4 x 1-3/4 inches].

          - Maximum Projection:  [3/8 inch] [1/2 inch].

          - Finish:  [26D Satin Chrome] [3 Bright Brass] [4 Satin Brass] [10B Oil Rubbed Bronze] [15 Satin Nickel] [19G Graphite black].

      - KEYING

        - Keying:  [Keyed different] [Keyed alike] [Master keying] [Grand master keying] [Construction keying] [Keyed to specific number].

- EXECUTION

      - EXAMINATION

        - Examine doors and frames, with Installer present, for compliance with requirements for installation tolerances, labeled fire door assembly construction, wall and floor construction, and other conditions affecting performance.

        - Examine roughing-in for electrical power systems to verify actual locations of wiring connections before electrified door hardware installation.

        - Proceed with installation only after unsatisfactory conditions have been corrected.

      - PREPARATION

        - Steel Doors and Frames:  Comply with DHI A115 series.

          - Surface-Applied Door Hardware:  Drill and tap doors and frames according to SDI 107.

        - Wood Doors:  Comply with DHI A115-W series.

      - INSTALLATION

        - Mounting Heights:  Mount door hardware units at heights indicated in following applicable publications, unless specifically indicated or required to comply with governing regulations:

          - Standard Steel Doors and Frames:  DHI's "Recommended Locations for Architectural Hardware for Standard Steel Doors and Frames."

          - Custom Steel Doors and Frames:  DHI's "Recommended Locations for Builders' Hardware for Custom Steel Doors and Frames."

          - Wood Doors:  DHI WDHS.3, "Recommended Locations for Architectural Hardware for Wood Flush Doors."

        - Installation:  Install each door hardware item to comply with manufacturer's written instructions.  Where cutting and fitting are required to install door hardware onto or into surfaces that are later to be painted or finished in another way, coordinate removal, storage, and reinstallation of surface protective trim units with finishing work.  Do not install surface-mounted items until finishes have been completed on substrates involved.

          - Set units level, plumb, and true to line and location.  Adjust and reinforce attachment substrates as necessary for proper installation and operation.

          - Drill and countersink units that are not factory prepared for anchorage fasteners.  Space fasteners and anchors according to industry standards.

        - Key Control System:  Place keys on markers and hooks in key control system cabinet, as determined by final keying schedule.

        - Boxed Power Supplies:  Locate power supplies as indicated or, if not indicated, above accessible ceilings. Verify location with Designer.

          - Configuration:  Provide the least number of power supplies required to adequately serve doors with electrified door hardware.

        - Thresholds:  Set thresholds for exterior and acoustical doors in full bed of sealant complying with requirements specified in Section 079200 - JOINT SEALANTS.

      - ADJUSTING

        - Initial Adjustment:  Adjust and check each operating item of door hardware and each door to ensure proper operation or function of every unit.  Replace units that cannot be adjusted to operate as intended.  Adjust door control devices to compensate for final operation of heating and ventilating equipment and to comply with referenced accessibility requirements.

          - Spring Hinges:  Adjust to achieve positive latching when door is allowed to close freely from an open position of 30 degrees.

          - Electric Strikes:  Adjust horizontal and vertical alignment of keeper to properly engage lock bolt.

          - Door Closers:  Adjust sweep period so that, from an open position of 70 degrees, the door will take at least 3 seconds to move to a point 3 inches from the latch, measured to the leading edge of the door.

        - Six-Month Adjustment:  Approximately six months after date of Substantial Completion, Installer shall perform the following:

          - Examine and readjust each item of door hardware as necessary to ensure function of doors, door hardware, and electrified door hardware.

          - Consult with and instruct the Owner's personnel on recommended maintenance procedures.

          - Replace door hardware items that have deteriorated or failed due to faulty design, materials, or installation of door hardware units.

      - CLEANING AND PROTECTION

        - Clean adjacent surfaces soiled by door hardware installation.

        - Clean operating items as necessary to restore proper function and finish.

        - Provide final protection and maintain conditions that ensure door hardware is without damage or deterioration at time of Substantial Completion.

      - HARDWARE SET SCHEDULE

        - Refer to the Drawings.

END OF SECTION